Terminal Server Navigation


>>>telnet citi.getmyip.com

password is cisco

term_server#r1

Trying r1 (192.168.1.5, 2065)... Open

R1
>

term_server#r2

Trying r2 (192.168.1.5, 2066)...

% Connection refused by remote host

term_server#sh line

   Tty Typ     Tx/Rx    A Modem  Roty AccO AccI   Uses   Noise  Overruns   Int

     0 CTY              -    -      -    -    -      2       0     0/0       -

*   65 TTY   9600/9600  -    -      -    -    -      7       1     0/0       -

*   66 TTY   9600/9600  -    -      -    -    -      3       6 15165/45498   -

    67 TTY   9600/9600  -    -      -    -    -      2       0     0/0       -

    68 TTY   9600/9600  -    -      -    -    -      2       0     0/0       -

    69 TTY   9600/9600  -    -      -    -    -      4      90   354/1060    -

    70 TTY   9600/9600  -    -      -    -    -      3       0    19/54      -

    71 TTY   9600/9600  -    -      -    -    -      1       4     1/0       -

Line(s) not in async mode -or- with no hardware support:

1-64, 97-128

term_server#clear line 66

[confirm]

 [OK]

term_server#r2

Trying r2 (192.168.1.5, 2066)... Open.

R2#

term_server#r3

Trying r3 (192.168.1.5, 2067)... Open

R3#

term_server#r4

Trying r4 (192.168.1.5, 2068)... Open

R4#

term_server#sh sess

Conn Host                Address             Byte  Idle Conn Name

   1 r1                  192.168.1.5            0     0 r1

   2 r2                  192.168.1.5           19     0 r2

   3 r3                  192.168.1.5            0     0 r3

*  4 r4                  192.168.1.5            0     0 r4

term_server#1

[Resuming connection 1 to r1 ... ]

�Hostname of device you wish to connect to


�Ctrl+Shift+6 then X is the break sequence to exit the connection to R1.


�This happens if connection is in use


�Shows async line status


�Clears line 66 to R2


�Establish connection to R2


�Shows active async sessions





�Resumes connection to R1 via session 1





